Press Release

The cone makes the difference:

Round cable entries for snap-in and screwing

Time is money – the versatile KES cable entry system from Conta-Clip saves valuable installation time, allowing users to equip control cabinets and enclosures in seconds. Conta-Clip continuously expands its range and has now added round cable entries with a cone for cables and wires without plugs for snap-in or screwing.

KES-R M Konus takes cables with outer diameters from 3 mm to 50 mm and is particularly suitable for applications where the exact cable diameters are not yet defined during design. The cable entries with their distinctive 12 corners are snapped directly from the outside into round, metric cutouts from M25 to M63. They fit in walls between 1.0 mm and 2.5 mm thick and ensure IP54 ingress protection. For larger wall thicknesses up to 11 mm and to achieve superior protection up to IP66, the same entries can be screwed tight with a lock nut.

In practical application, after determining the cable diameter the cable entry is opened with a clean cut so that the diameter marking indicated on the cone step corresponds to the cable to be inserted and remains visible. The silicone-free and halogen-free material mix of TPE and fiberglass-reinforced polyamide 6.6 ensures tightness and stability. The KES-R M Konus series comprises five sizes for metric cutouts from M25 to M63. This allows flexible and reliable sealing of cable diameters from 3 mm to 50 mm.

Caption: A quick cut, and the cable entries with a cone take any cables between 3 mm and 50 mm

| Image/s: | KES-R_M_Konus_2000px.jpg | Characters: | 1376 |
| --- | --- | --- | --- |
| File name: | 202511028_pm_kes-r_m_konus_cable_entries_en | Date: | 11-24-2025 |
| Tags: | cable entry, cable entries, M25, M63, control cabinets, enclosures |  |  |
| About CONTA-CLIP CONTA-CLIP is one of Europe’s leading manufacturers of electrical and electronic connection elements and cable management solutions. Based in Hövelhof, Germany, the medium-sized, family-run company has been producing electric and electronic connection systems for the process and automation industries since 1978. CONTA-CLIP supplies all industry sectors, focusing on railroad industries, shipbuilding, building automation, conveyor technology, machine and plant engineering and construction, instrumentation and control technology, control panel manufacturing, transformer manufacturing, and environmental technology. The company consists of many specialized branches: CONTA-CONNECT for electric connection technology, CONTA-CABLE for cable management systems, CONTA-ELECTRONICS for electronics and CONTA-CON for PCB edge connectors. Additionally, CONTA-CLIP provides services such as customizing housings and mounting rails, labeling and supplying user-specific electronic components. |  |  |  |
| Contact: CONTA-CLIP Verbindungstechnik GmbH Christian Quade Otto-Hahn-Str. 7 33161 Hövelhof Germany Phone: +49 . 5257 . 9833 - 172 Email: christian.quade@conta-clip.de Internet: www.conta-clip.com |  | gii die Presse-Agentur GmbH Christburger Str. 41 10405 Berlin Germany Phone: +49 . 30 . 538 965 - 0 Email: info@gii.de Internet: www.gii.de |  |